# System Requirements Specification (SRS)

Product: Automotive Battery Control Unit (BCU-800V)
Document Ref: SRS-AUT-BCU-800V | Revision: 3.2 | Status: Released
System Architecture: 800V Silicon-Carbide EV Platform
Applicable Standards: ISO 26262 (ASIL D), ISO 6469-3, UN ECE R100, ISO 21434, IEC 60529

# 1. Purpose and System Architecture Overview

The BCU-800V is an automotive safety-critical Electronic Control Unit (ECU) responsible for monitoring, state estimation, contactor sequencing, thermal supervision, and high-voltage isolation of the 800V traction battery system. This specification defines all mandatory electrical, safety, environmental, and cybersecurity requirements for vehicle type approval.

# 2. Technical Requirements Register

REQ-BCU-001: High Voltage Pack Nominal Operating Range
Severity: Critical | Category: Electrical
The BCU shall monitor and regulate high-voltage traction packs operating from 400.0 V DC to 800.0 V DC under all standard driving and fast-charging conditions.

REQ-BCU-002: Continuous Discharge Current Monitoring
Severity: High | Category: Electrical
The primary current shunt sensing stage shall accurately measure continuous discharge currents up to 350.0 A with maximum error <= 0.5% full-scale.

REQ-BCU-003: Galvanic High-Voltage Isolation Barrier
Severity: Critical | Category: Safety
The galvanic isolation barrier between high-voltage traction circuitry and 12V low-voltage logic domain shall withstand 2.5 kV AC RMS for 60 seconds with leakage current < 1.0 mA.

REQ-BCU-004: CAN-FD Telemetry Communication Rate
Severity: Medium | Category: Electrical
The powertrain telemetry interface shall support ISO 11898-2 CAN-FD protocol at 5.0 Mbps data phase rate with zero frame errors during full bus load.

REQ-BCU-005: Quiescent Sleep State Current Draw
Severity: Medium | Category: Electrical
When vehicle ignition is off and BCU enters deep sleep mode, total quiescent current from the 12V auxiliary battery shall not exceed 150.0 uA.

REQ-BCU-006: Cell Voltage Sensing Measurement Precision
Severity: High | Category: Electrical
Individual battery cell voltage measurement accuracy shall be within +/- 1.5 mV across the entire 2.0 V to 4.5 V cell chemistry operating window.

REQ-BCU-007: Safe State Contactor De-energization Latency
Severity: Critical | Category: Safety
Upon detection of critical overvoltage or overtemperature fault, the safety supervisor shall open main DC contactors within maximum latency <= 10.0 ms.

REQ-BCU-008: Ingress Protection Rating IP67
Severity: High | Category: Mechanical
The BCU aluminum die-cast housing and sealed automotive connectors shall satisfy IP67 ingress protection per ISO 20653 / IEC 60529.

REQ-BCU-009: Pre-Charge Sequence Completion Time
Severity: Medium | Category: Electrical
The DC link pre-charge circuit sequence shall charge inverter DC-link capacitance to 95% of pack voltage within maximum duration <= 200.0 ms.

REQ-BCU-010: Solid-State Pyro-Fuse Trigger Circuit Latency
Severity: Critical | Category: Safety
In hard short-circuit conditions, the solid-state pyro-fuse ignition circuit shall issue trigger pulse in <= 5.0 microseconds to isolate traction battery.

REQ-BCU-011: Extended Climatic Operating Temperature
Severity: High | Category: Environmental
The BCU shall maintain full functional performance and sensor accuracy across ambient operating temperature range from -40.0 °C to +85.0 °C.

REQ-BCU-012: Damp Heat Cyclic Humidity Endurance
Severity: Medium | Category: Environmental
The controller shall withstand damp heat cyclic testing at 95.0% relative humidity and +65.0 °C for a continuous duration of 500.0 hours without insulation degradation.

REQ-BCU-013: Thermal Runaway Gas Venting Pressure Calculation and Burst Validation
Severity: Critical | Category: Safety
The pack pressure relief membrane must calculate thermal runaway gas release dynamics and demonstrate physical burst opening at pressure <= 300.0 mbar.

REQ-BCU-014: Tri-Axial Random Vibration Endurance Profile
Severity: High | Category: Mechanical
The unit shall withstand random vibration per ISO 16750-3 across all three orthogonal axes (X, Y, and Z) for 32 hours per axis at 2.8 g RMS.

REQ-BCU-015: Overcurrent Protection Multi-Tier Inverse Time Curve
Severity: High | Category: Electrical
Overcurrent trip response shall verify the complete multi-tier curve spanning 100.0 A to 500.0 A with programmable inverse-time characteristics.

REQ-BCU-016: Contact Resistance Degradation After Power Cycling
Severity: Medium | Category: Electrical
Terminal power lug contact resistance shall remain <= 0.20 mOhm after 100,000 power cycles as verified by micro-ohmmeter measurement record.

REQ-BCU-017: ISO 21434 Hardware Root-of-Trust Secure Boot Signature
Severity: Critical | Category: Cybersecurity
The BCU microcontroller shall enforce secure boot verification with ECDSA P-384 hardware root-of-trust key validation to prevent unauthorized firmware execution.

REQ-BCU-018: High Voltage Interlock Loop (HVIL) Fast Disconnect Latency
Severity: Critical | Category: Safety
Any break or interruption in the continuous high-voltage interlock loop shall trigger active discharge and contactor opening in <= 5.0 ms.

REQ-BCU-019: High Altitude Barometric Pressure Operation
Severity: Medium | Category: Environmental
The BCU shall operate without high-voltage corona discharge or thermal overheating at altitudes up to 4000.0 m (61.6 kPa atmospheric pressure).

REQ-BCU-020: Salt Spray Atmosphere Corrosion Resistance
Severity: Medium | Category: Mechanical
External connectors and housing shall show no galvanic corrosion or pitting after 720.0 hours neutral salt spray test per ASTM B117 / ISO 9227.

REQ-BCU-021: Radiated RF Electromagnetic Immunity 100 V/m
Severity: High | Category: Electrical
The BCU shall maintain uninterrupted operation during radiated RF field immunity exposure of 100.0 V/m from 200.0 MHz to 2.0 GHz per ISO 11452-2.

REQ-BCU-022: Mean Time Between Failures Reliability MTBF Target
Severity: Medium | Category: Documentation
Theoretical system reliability MTBF shall exceed 250,000 hours continuous operation at 40°C ambient calculated per Telcordia SR-332 / SN 29500.

REQ-BCU-023: State of Charge (SOC) Extended Kalman Filter Estimation Accuracy
Severity: High | Category: Documentation
The battery state-of-charge estimator shall maintain SOC tracking drift < 2.0% across dynamic WLTP and US06 drive cycles over 1,000 operating hours.

REQ-BCU-024: Upper Limit Traction Pack Voltage Rating
Severity: Critical | Category: Electrical
The BCU sensing and isolation design shall operate across 400.0 V DC to 800.0 V DC traction bus voltage envelope.

REQ-BCU-025: Maximum Ambient Operating Thermal Limit
Severity: High | Category: Environmental
The power electronics stage shall operate up to +85.0 °C ambient temperature under continuous current load.

REQ-BCU-026: Diagnostic Service Port Access Authentication
Severity: High | Category: Cybersecurity
Unified Diagnostic Services (UDS) over Ethernet shall mandate cryptographic seed-key authentication before permitting flashing or calibration access.

REQ-BCU-027: Galvanic Isolation vs Ground Architecture
Severity: Critical | Category: Safety
The high-voltage analog measurement sub-circuit shall maintain complete galvanic isolation from the vehicle chassis ground.

REQ-BCU-028: Active Cell Balancing Discharge Current
Severity: Medium | Category: Electrical
Active cell balancing circuit shall deliver balancing current of 200.0 mA per channel to equalize pack state-of-charge.

REQ-BCU-029: Electrolyte Leakage Detection Acoustic Impedance Threshold
Severity: High | Category: Safety
The ultrasonic acoustic sensing channel shall flag electrolyte leakage when acoustic signature dampening exceeds 12.0 dB relative to dry baseline.

REQ-BCU-030: High Voltage Pyro-Switch Minimum Firing Energy
Severity: Critical | Category: Safety
Capacitive firing module shall store minimum 4.5 J energy at 24V booster stage to guarantee single-pulse detonation of the main safety disconnector.